[NOTE: This is a slightly modified version of Chapter 2 of the PhD thesis of Patrick M. Len (Department of Physics, University of California, Davis, June 1997).  Section 2.2 has been replaced to refer to a more recent and advanced program to calculate photoelectron diffraction intensities, MSCD.  The figures are available only as ClarisDraw files.  The program listings are omitted.]

Overview of Theoretical Methodology and

Numerical Programs Used in Atomic Holography

Abstract

This chapter briefly describes the theoretical methodology and the FORTRAN77 programs used in this study of atomic electron and x-ray holography, covering in more detail programs that numerically reconstruct atomic images from normalized holographic  intensity data sets.

Outline

2.1.  Introduction

2.2.  Programs to calculate holographic photoelectron diffraction intensities

2.3.  Programs to calculate holographic x-ray diffraction intensities

2.4.  Programs to normalize raw holographic intensities

2.5.  Programs to reconstruct atomic images

2.6.  Modifications to the HoloInvert.f main subroutines to treat other imaging methods

2.7.  Graphical presentation of reconstructed atomic images

2.8.  Concluding remarks

2.1  Introduction

In the course of developing atomic electron and x-ray holography as a practical probe of atomic structure, various programs have been developed to generate holographic electron (ssc-vib.f, scat.f) and x-ray (Xray-holo.f) diffraction pattern intensities.  Since in practice the reference wavefront  cannot be analytically known and must be deduced from the experimental raw holographic I(k) intensities, several programs to extract  and subsequently calculate the normalized holographic  intensities have also been developed (Io-magk.f, Io-khat.f, Filter.f, Fit-Io.m).  Finally, despite the aesthetic allure of experimentally illuminating such holographic diffraction patterns with a reference wavefront in order to recover visible real-space atomic images, programs to numerically reconstruct atomic images in a more convenient and practical manner have been developed (HoloInvert.f), given the computing power readily available on supercomputers, workstations, and more recently, desktop personal computers.

Some of these programs were developed prior to the present study (ssc-vib.f, scat.f), but most were written by the author, and/or represent significant modifications of earlier programs.

2.2  Programs to calculate holographic photoelectron diffraction intensities

[Start of modified text]

MSCD

MSCD is a program that calculates photoelectron diffraction intensities which can be used as simulated holographic intensities.  It can be downloaded for free at http://electron.lbl.gov/software/mscdpack/, where full documentation is also available.

[End of modified text]

2.3  Programs to calculate holographic x-ray diffraction intensities

Xray-holo.f

Xray-holo.f is a FORTRAN77 program that calculates normalized holographic x-ray  intensities from single-scattering model clusters.  This code was originally adapted by Thevuthasan from the electron holographic I(k) intensity program ssc-vib.f [2.1], as the physics that describes x-ray scattering is a much simplified analog of the single-scattering model of electron diffraction.  Specifically, the photoexcitation cross-section and photoemitted source wave are assumed to be isotropic for the x-ray cases studied in this work, and no Debye-Waller temperature effects are included.  Isotropic x-ray fluorescence should be rigorously true in the case of the Ka transitions considered in this dissertation, since the initial hole is in the spherically-symmetric 1s level.  The neglect of Debye-Waller effects can also be justified, as the main effect of sample temperature is to reduce the contrast of holographic diffraction fringes, mainly from backscattering atoms [2.11].  However, a sufficient number of counts can be made to compensate for the detrimental effect of Debye-Waller vibrations on contrast, as demonstrated by room temperature experiments [2.12-2.14].  For Mo, with an atomic weight of 95.94g/mol, and a Debye temperature of 400K, the mean squared displacement at T = 300K is thus:

(2.7(a))

which then leads to the following Debye-Waller factors for forward (), side () and backscattering () geometries for Mo Kb x-rays ():

(2.6(a))

Thus, provided that contrast problems can be resolved, Debye-Waller attenuation of diffraction is not expected to otherwise affect reconstructed atomic images, as this effect ranges less than an order of magnitude over all scattering directions.

Inelastic attenuation of x-rays is also expected to be negligible in our calculations for several reasons.  First, the attenuation lengths are very long (» mm).  Also, the Borrmann effect of anomalous non-dispersive scattering [2.15] is disregarded, as considering a cluster large enough for such an effect to become important  would be too computationally unwieldy (, assuming ) for even supercomputers.  Finally, we are primarily interested in imaging the short-range atomic structure around a given emitter, and so the inclusion of such distant scatterers has no effect on these components of the resulting hologram [2.12].

Xray-holo.f consists of a family of related programs (Progs. 2.1-2.3) to calculate x-ray holographic  intensities for a number of different experimental geometries:

Table 2.1.  Xray-holo.f program family

| Xray-holo.f | XFH, or MEXH with unpolarized incident radiation |
| --- | --- |
| Xray-holo-p1.f | MEXH with horizontally polarized incident radiation |
| Xray-holo-p2.f | MEXH with vertically polarized incident radiation |

Input file format

All the Xray-holo.f programs use the same input file and format (Xray-holo.in):

Example 2.1.  Xray-holo.in sample file

| Fe-bcc:0d | name of atomic cluster coordinate file |
| --- | --- |
| Fe:sf | name of x-ray scattering factor file |
| 10. | º starting polar angle (in degrees) as measured with respect to the sample surface |
| 5. | º size of polar angle steps (in degrees) |
| 16 | º number of polar angle steps (in degrees) |
| 0. | º starting azimuthal angle (in degrees) |
| 5. | º size of azimuthal angle steps (in degrees) |
| 72 | º number of azimuthal angle steps (in degrees) |
| 8.0 | º starting wavenumber (in ) |
| 0.1 | º size of wavenumber steps (in ) |
| 11 | º number of wavenumber steps (in ) |
| Fe-bcc:xK | name of normalized holographic file |

Of note are the parameters that specify at which points in k-space  to calculate normalized holographic x-ray  intensities.  These points are equally spaced with respect to wavenumber () and angular  intervals.  Polar angle q is measured from the plane of the surface (i.e.,  is the surface normal).

Two data files are subsequently read in by Xray-holo.f: the atomic cluster coordinate file, and the x-ray scattering factor file.

X-ray scattering factor file

This data file is a listing of the real and imaginary components of the complex x-ray scattering factor , where Q is the angle between the incident (k) and scattered (k') wavefronts,  is the scattering angle-dependent component of the scattering factor, and  and  are the dispersive-specific components of the scattering factor.  These parameters can be adapted from published tables [2.16] using commercial or public domain interpolation applications (e.g., MathematicaÓ [2.17], spline [2.18], etc.).  The published tables usually give values of , in relative units of the electron scattering strength, i.e., the Thomson scattering factor, which is for unpolarized incident radiation [2.19]:

Tunpolarized(Θ)≡dσdΩ=e2mc2(1+cos2Θ)2.						  (2.10)

For polarized incident radiation, the Thomson scattering factor for a single electron is given by:

Tpolarized(Θek')≡dσdΩ=e2mc2sin2Θek',						  (2.11)

where  is the angle between the polarization vector of the incident radiation , and the direction k' of the scattered radiation.

In this dissertation, the raw  table values have been interpolated to a wavenumber and scattering angle dependent scattering factor  for , using the Interpolate[] function of MathematicaÓ [2.17], and setting

,									  (2.12)

,									  (2.13)

in order to ensure smooth and unkinked forward and backscattering peaks.  Also listed in Ref. [2.16] are the real and imaginary parts of the dispersive contributions to the scattering factor,  and , which are incorporated into the complete complex scattering factor as .  The scattering factor for a given wavenumber k is finally listed at  intervals from  to ; different wavenumber scattering factors (as specified in Xray-holo.in) are then listed sequentially:

Example 2.2.  Scattering factor file format

| Wavenumber (in ) | Scattering angle (in 1° intervals) | Real part of scattering factor (in electrons) | Imaginary part of scattering factor (in electrons) |
| --- | --- | --- | --- |
| k | Q° |  |  |

All scattering factor values are in relative units of electrons, which are then converted to absolute scattering factor units within Xray-holo.f, depending on the polarization of the incident x-rays, as discussed above.  Figure 2.2(a) shows the atomic scattering factor for a single Fe atom for  x-rays, in relative electron units.  Figures 2.2(b)-(d) shows the Thomson scattering factor for a single electron incident with (b) unpolarized, (c) horizontally polarized, and (d) vertically polarized incident radiation.  For the three programs in Table 2.1, the final scattering factors are calculated from multiplying the electron unit scattering factor  with the appropriate Thomson scattering factor, whether for unpolarized (Eq. (2.10) and Fig. 2.2(b)) incident radiation, or polarized (Eq. (2.11) and Figs. 2.2(c)-(d)) incident radiation.  Scattering angles Q and polarization angles  for each emitter-scatterer pair are then determined within Xray-holo.f, in order to linearly interpolate the relative  scattering factor, or to calculate the appropriate Thomson scattering factor, respectively.

Cluster coordinate file

Common to ssc-vib.f, the cluster coordinate file lists the  coordinates of each atom, whether it is an emitter (code = 1) or scatterer (0), and its type (always 1, since we have so far considered only arrays of one kind of emitter and scatterer).  The emitter need not be at the origin, as relative emitter-scatterer vectors are calculated in Xray-holo.f.  Example 2.3 shows an Fe[001] trimer coordinate file, with a single emitter in the middle of two scatterers oriented perpendicular to a hypothetical Fe(001) surface.

Example 2.3.  Fe[001] trimer

| x (in Å) | y (in Å) | z (in Å) | emitter (1) or scatterer (0) | species type (º 1) |
| --- | --- | --- | --- | --- |
| 0.00 | 0.00 | 2.96 | 0 | 1 |
| 0.00 | 0.00 | 0.00 | 1 | 1 |
| 0.00 | 0.00 | -2.96 | 0 | 1 |

In the current implementation of Xray-holo.f, only clusters containing a single emitter and atomic species can be considered.  Should either more than one unique emitter site, or more than one scattering atomic species needs to be considered, then the following procedure has been used as an alternative to significant reworking of Xray-holo.f  A single modification can be made to Xray-holo.f such that raw complex  amplitudes are separately output for each nth emitter/scatterer combination.  Then each of these  amplitudes are coherently summed to obtain the holographic intensities , from which normalized holographic intensities  are obtained, where N is the total number of different emitter/scatterer combinations involved, and  has been normalized to unity.

Xray-holo.f main subroutine

The main subroutine of Xray-holo.f is a multiply nested loop that calculates normalized holographic  intensities over different wavenumbers (k) and directions  in k-space, where the innermost nested loop in curly brackets is the single scattering convolution integral of Eqs. (1.8) and (1.4(c)), for discrete point scatterers located at :

(2.14)

Xray-holo.f output format

Xray-holo.f writes out normalized holographic  intensities for consecutive increasing wavenumber (k), polar angle (q), and azimuthal angle (f) values.

Example 2.4  Xray-holo.f output format

| Wavenumber (in ) | Polar angle (in degrees) | Azimuthal angle (in degrees) | Normalized holographic intensity |
| --- | --- | --- | --- |
| k | q | f |  |

2.4  Programs to normalize raw holographic intensities

Three FORTRAN77 programs, along with a MathematicaÓ macro, calculate the normalized holographic  intensities from raw holographic I(k) intensities via:

,									  (2.15)

where the methods by which the background  is calculated are discussed in detail below:

Table 2.2.  Holographic data normalization programs

| Io-magk.f | Normalizes each different set of variable-energy I(k) data points separately by subtraction of an average constant background |
| --- | --- |
| Io-khat.f | Normalizes each different set of variable-direction I(k) data points separately by removal of a linear background |
| Filter.f | Normalizes each different set of variable-energy data points separately by removal of low-frequency components |
| Fit-Io.m | Normalizes with respect to both energy and polar angle by removal of a low-order polynomial fit |

Io-magk.f

Io-magk.f (Prog. 2.4) normalizes raw I(k) intensities by calculating  as an average value for each set of different scanned-energy (scanned-wavenumber) data points , averaging over directions at each different wavenumber k [2.20]:

= k2dσkIk(k)k2dσk.									  (2.16)

Io-khat.f

Io-khat.f (Prog. 2.5) normalizes raw I(k) intensities by calculating  as a linear fit for each set of scanned-energy (scanned-wavenumber) data points  at a given direction  [2.21]:

= ,									  (2.17)

Filter.f

Filter.f (Prog. 2.6) normalizes raw I(k) intensities by calculating  as the low-frequency angular components for each different scanned-angle I(k) data set at a given energy (wavenumber), as suggested by Harp et al. [2.22]:

= k2dσke−σ2(k−k')2Ik(k')k2dσk							  (2.18)

where s is a variable parameter which smooths the original I(k) over a certain length scale in ||.  Thus data points at I(k) are smoothed with data located at relative positions (), and these other data points I(k') are weighted by the Gaussian function .  Note that as s®0, this procedure degenerates into the Io-magk.f normalization scheme.  Filter.f has been found to be most appropriate to find  when the most significant correction to the raw I(k) intensities is the removal of low-angular frequency geometric absorption effects in x-ray holograms [2.13].  Equation (2.5) has also been used to remove high-frequency scattering components in x-ray fluorescence holograms [2.12].  For electron holographic I(k) intensities, this method has been used to remove localized forward scattering features [2.22,2.23], and a version of Filter.f has been modified by Ruebush to specifically remove high-angular frequency noise from experimental electron diffraction patterns [2.24].

Input file format

All of the above normalization programs (Io-magk.f, Io-khat.f, Filter.f) use the same input file and format (Io.in):

Example 2.2.  Io.in sample file

| Fe-bcc:Ik | raw I(k) intensity file (arb. units) |
| --- | --- |
| Fe-bcc:xK | normalized intensity file (arb. units) |
| 72 | º number of polar angle steps (in degrees) |
| 37 | º number of azimuthal angle steps (in degrees) |
| 0.0 | º number of wavenumber steps (in ) |
| 0.15 | s (for Filter.f) (in Å) |

Fit-Io.m

Fit-Io.m (Prog. 2.7) is a MathematicaÓ macro file that enables a raw I(k) data set to be read in and normalized by the MathematicaÓ kernel, where  is determined by:

EMBED Equation  = .						  (2.19)

This three-dimensional k-space polynomial fit with respect to both wavenumber k and polar angle q is CPU intensive, as typical I(k) and  data sets comprise of up to 103-104 intensities.  Such data sets are normalized using Fit-Io.m in 102-103 minutes of CPU time on a Sun SPARC2 workstation.  The number of coefficients (above, 1 £ m,n £ 6) can be truncated or expanded as need arises.  The number of coefficients needed to adequately normalize I(k) intensities can be determined by the inspection and imaging of the  and  intensities that are determined in the normalization process.  Normally, the above sums should be restricted to  to avoid removing valid diffraction features from the data set.  Figure 2.3(a) shows a raw theoretical I(k) data set calculated for surface W 4f photoemission from a multiple-scattering W(110) cluster (cf. Chapter 5).  The theoretical background , and normalized  data sets corresponding to Fig. 2.3(a) are shown in Fig. 2.3(b)-(c), respectively.  Since the background  cannot be known analytically (unless a theoretical I(k) data set is considered, as here), Figs. 2.4(b)-(c) and 2.5(b)-(c) show progressively higher-order wavenumber and polar angle fits of Eq. (2.19) to the raw theoretical I(k) data set of Fig. 2.3(a).  For comparison, the theoretical background  is shown in Figs. 2.4(a) and 2.5(a).  As can be seen in Figs. 2.4-2.5, the lower order polynomial fits of (m,n) £ (1,1) (Fig. 2.4(b)) and (m,n) £ (2,2) (Fig. 2.4(c)) do not adequately reproduce the fine structure features of the theoretical background  of Fig. 2.4(a).  However, the (m,n) £ (3,3) polynomial fit of Fig. 2.5(b) generally reproduces the bright feature seen at  in Fig. 2.5(a), as does the (m,n) £ (6,6) polynomial fit of Fig. 2.5(c), which also reproduces the azimuthal bands seen in Fig. 2.5(a).  However, caution should be exercised when extending the polynomial fit to higher orders, as finer features that will begin to be included in  may actually correspond to holographic modulations.  The best methodology is simply to reconstruct images from progressively higher order polynomial fit normalized  data sets, until these images no longer appreciably improve in fidelity.

Table 2.3 shows the  coefficients for the polynomial  fit to the theoretical surface emission I(k) of Fig. 2.1(a), where  has been normalized to 1.

Table 2.3  Coefficients for

|  | n = 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m = 0 | -1.0 | 1.8 | 3.4 | 1.6 | 0.34 | 1.6 | -0.021 |
| 1 | 0.54 | 0.83 | -1.5 | 0.75 | 0.19 | 0.74 | -0.079 |
| 2 | 0.73 | -1.4 | 3.1 | -1.3 | -0.22 | -1.3 | 0.22 |
| 3 | 0.22 | 0.73 | -1.2 | 0.63 | 0.12 | 0.56 | -0.096 |
| 4 | 0.025 | -0.21 | 0.14 | 0.16 | -0.45 | -0.11 | 0.017 |
| 5 | 8.4e-4 | 0.032 | -1.3e-3 | 0.022 | 7.9e-3 | 0.011 | -1.2e-3 |
| 6 | 3.1e-5 | 1.8e-3 | 5.5e-3 | -1.2e-3 | 4.9e-4 | 4.7e-4 | 2.0e-5 |

In Table 2.3, the  coefficients for  are on the order of unity, while those for  are for the most part an order of magnitude less.  However, it also appears from the entries that it may be necessary to take n higher than m, and up to as high as 5.  But this should be tested for each case using the image criterion mentioned above.

The file names of the raw I(k), background , and normalized  data sets must be explicitly written into the macro file, as well as the number of k, q, and f steps in these data sets.

2.5  Programs to reconstruct atomic images

HoloInvert.f

HoloInvert.f is a FORTRAN77 program that reconstructs image intensities from either normalized electron or x-ray holographic  intensities.  HoloInvert.f consists of a family of related programs to calculate image U(r') intensities via different reconstruction algorithms:

Table 2.4.  HoloInvert.f program family

| HoloInvert.f | Method A reconstruction algorithm |
| --- | --- |
| HoloInvert:2.0a.f | Method reconstruction algorithm |
| HoloInvert:2.0b.f | Method B reconstruction algorithm |
| HoloInvert:2.0c.f | Method C reconstruction algorithm |
| HoloInvert:2.0d.f | Method D reconstruction algorithm |

Input file format

All the HoloInvert.f programs use the same input file and format (HoloInvert.in):

Example 2.3  Sample HoloInvert.in format for three-dimensional volume reconstruction

| Fe-bcc:xK |  |  | normalized hologram file (in arb. units) |
| --- | --- | --- | --- |
| 16 |  |  | º number of polar angle steps (in degrees) |
| 72 |  |  | º number of azimuthal angle steps (in degrees) |
| 10 |  |  | º number of wavenumber steps (in ) |
| 0.0 |  |  | rotation angle (in degrees) |
| Fe-bcc:xyz:Ur |  |  | image intensity U(r') (in arb. units) |
| -6.0 | 6.0 | 0.1 | (in Å) |
| -6.0 | 6.0 | 0.1 | (in Å) |
| -6.0 | 6.0 | 0.1 | (in Å) |

Of note are the parameters that specify at which points in real space (r' = (x',y',z')) to calculate image U(r') intensities.  The 1, 2, or 3 dimensional region to be covered in a given reconstruction is specified by the appropriate beginning and ending ,  limits (highlighted in boldface).  Example 2.3 above is thus for a reconstruction covering a volume of ±6.0 Å in all three directions.  Examples 2.4-2.5 show image reconstruction parameters for the [100] axis and the (001) plane, respectively.

The rotation angle  is defined as the counterclockwise rotation angle of the hologram with respect to the fixed reconstruction axes x', y', and z', which in effect rotates reconstructed images with respect to the real-space region being reconstructed.  This feature allows the alignment of images reconstructed from  data that is not already aligned azimuthally with respect to a low-index direction (e.g., [100]).

Example 2.4  Sample HoloInvert.in format for [100] axis reconstruction

| Fe-bcc:xK |  |  | normalized hologram file (in arb. units) |
| --- | --- | --- | --- |
| 16 |  |  | º number of polar angle steps (in degrees) |
| 72 |  |  | º number of azimuthal angle steps (in degrees) |
| 10 |  |  | º number of wavenumber steps (in ) |
| 0.0 |  |  | rotation angle (in degrees) |
| Fe-bcc:x100:Ur |  |  | image intensity U(r') (in arb. units) |
| -6.0 | 6.0 | 0.1 | (in Å) |
| 0.0 | 0.0 | 0.1 | (in Å) |
| 0.0 | 0.0 | 0.1 | (in Å) |

Example 2.5  Sample HoloInvert.in format for (001) plane reconstruction

| Fe-bcc:xK |  |  | normalized hologram file (in arb. units) |
| --- | --- | --- | --- |
| 16 |  |  | º number of polar angle steps (in degrees) |
| 72 |  |  | º number of azimuthal angle steps (in degrees) |
| 10 |  |  | º number of wavenumber steps (in ) |
| 0.0 |  |  | rotation angle (in arb. units) |
| Fe-bcc:xy001:Ur |  |  | image intensity U(r') |
| -6.0 | 6.0 | 0.1 | (in Å) |
| -6.0 | 6.0 | 0.1 | (in Å) |
| 0.0 | 0.0 | 0.1 | (in Å) |

The image region of interest need not be either centered, nor pass through the origin.  Example 2.6 shows the parameters for imaging the (002) plane of Fe(001) that is 1.453 Å above the emitter plane:

Example 2.6  Sample HoloInvert.in format for (002) plane reconstruction

| Fe-bcc:xK |  |  | normalized hologram file |
| --- | --- | --- | --- |
| 16 |  |  | º number of polar angle steps (in degrees) |
| 72 |  |  | º number of azimuthal angle steps (in degrees) |
| 10 |  |  | º number of wavenumber steps (in ) |
| 0.0 |  |  | rotation angle |
| Fe-bcc:xy001:Ur |  |  | image intensity U(r') |
| -6.0 | 6.0 | 0.1 | (in Å) |
| -6.0 | 6.0 | 0.1 | (in Å) |
| 1.435 | 1.435 | 0.1 | (in Å) |

In this manner, any directions of the form [h00], [0k0], [00l], or planes of the form (h00), (0k0), (00l) can be imaged.

By specifying the (counterclockwise) rotation angle  of the hologram with respect to the reconstructed image reference frame, all directions of the form [hk0], as well as planes of the form (hk0) can be imaged.  Examples 2.7-2.8 show input parameters for the (110) and (120) planes for bulk Fe(001), respectively:

Example 2.7  Sample format for (110) plane reconstruction

| Fe-bcc:xK |  |  | normalized hologram file (in arb. units) |
| --- | --- | --- | --- |
| 16 |  |  | º number of polar angle steps (in degrees) |
| 72 |  |  | º number of azimuthal angle steps (in degrees) |
| 10 |  |  | º number of wavenumber steps (in ) |
| 45.0 |  |  | rotation angle (in degrees) |
| Fe-bcc:xz110:Ur |  |  | image intensity U(r') (in arb. units) |
| -6.0 | 6.0 | 0.1 | (in Å) |
| -6.0 | 6.0 | 0.1 | (in Å) |
| 0.0 | 0.0 | 0.1 | (in Å) |

Example 2.8  Sample format for (120) plane reconstruction

| Fe-bcc:xK |  |  | normalized hologram file (in arb. units) |
| --- | --- | --- | --- |
| 16 |  |  | º number of polar angle steps (in degrees) |
| 72 |  |  | º number of azimuthal angle steps (in degrees) |
| 11 |  |  | º number of wavenumber steps (in ) |
| 26.56505118 |  |  | rotation angle (in degrees) |
| Fe-bcc:xz120:Ur |  |  | image intensity U(r') |
| -6.0 | 6.0 | 0.1 | (in Å) |
| -6.0 | 6.0 | 0.1 | (in Å) |
| 0.0 | 0.0 | 0.1 | (in Å) |

Image reconstructions in general [hkl] directions or (hkl) planes are not supported in this version of HoloInvert.f; though such directions/planes would be possible through the incorporation of full Euler rotation operations on the orientation of the hologram, with respect to the image reconstruction reference frame.

Note that image resolutions (dx',dy',dz') need not be identical for each different direction, as long as they are non-zero.  This stems from the nature of the algorithm that determines from the input parameters at which r' = (x',y',z') points the image U(r') intensities are to be calculated.

Hologram data file format

The hologram data file format is identical to the hologram data output format from Xray-holo.f, where normalized holographic  intensities are listed for consecutive increasing wavenumber (k), polar angle (q), and azimuthal angle (f) values.  Note that the data is explicitly in the format of an  matrix; any missing values in k-space must be explicitly listed as zero.

Example 2.9  Hologram data file format

| Wavenumber (in ) | Polar angle (in degrees) | Azimuthal angle (in degrees) | Normalized holographic intensity |
| --- | --- | --- | --- |
| k | q | f |  |

Also important is that the  data intervals for wavenumber () and angular (, ) spacings must be identical for all entries.  Numerous commercial programs can be used to interpolate holographic image intensity data sets onto a constant  k-space lattice if this is not initially the case.  One that was used in this dissertation is the Fit[] function of MathematicaÓ [2.17].

HoloInvert.f main subroutine

The main subroutine of HoloInvert.f (Prog. 2.8) is a multiply nested loop containing the numerical evaluation of the reconstruction algorithm of Method A (Eq. (1.9)) for image intensities located at r':

.						  (2.20)

In actually representing the final image, it is the absolute value of this quantity that is plotted over the r' space; the same is true for the other images , B, and C discussed below.  Because q is here defined as a takeoff angle relative to the surface instead of the often-used polar angle q' = 90° - q measured relative to the normal, we note that the factor  in this sum is just the non-constant factor in a volume element in k-space of the form .

The wavenumber summation is the innermost loop, such that the geometric path length difference  needs to be evaluated only once for each different (q,f) direction in the  data set for each image intensity at r'.

It should be noted on closer inspection that HoloInvert.f is neither fully vectorized nor optimized, but it nonetheless already performs moderately well on Sun SPARC2 workstations, evaluating  data point  holograms to generate  image U(r') intensities within  minutes.  Performance on CRAY supercomputers indicates at least an order of magnitude or better increase in evaluation times.  The motivation behind the implementation of HoloInvert.f was to translate Eq. (2.5) into FORTRAN77 using the most straightforward and transparent code.  Use of more sophisticated integration summation methods (e.g. trapezoidal rule, Simpson's rule, etc.), was not found to significantly improve image fidelity.  Thus, the simple Reimann integration summation used here is adequate to reconstruct image U(r') intensities of sufficient fidelity in HoloInvert.f, while keeping this code as simple and compact as possible for any further modifications.

2.6  Modifications to the HoloInvert.f main subroutines to treat other imaging methods

Method

For Method  (Prog. 2.9), an additional data file is read in by HoloInvert:2.0a.f: the scattering factor file fk.in, which has the same format as the scattering factor files read by Xray-holo.f (cf. Ex. 2.2).

The main numerical evaluation subroutine of the reconstruction algorithm of Method  (Eq. (1.10)) then becomes:

,					  (2.21)

where the scattering factor  is interpolated via a linear interpolation subroutine (cf. Xray-holo.f) for each unique scattering angle Q between r' and k.  There is no dependence on the radiation polarization here, and so this program cannot be used to correct fully for the more complex x-ray case in which Thomson scattering of polarized incoming radiation may have to be allowed for.  (An alternative method to correct for this effect is discussed in Chapter 8).  Note also that this method requires knowing the identity of the atom at each site in order to specify .  If only one type of atom is present, this is trivial, but if not, then some sort of trial and error procedure and/or prior knowledge of approximate atomic positions is required.

Method B

The additional data file read in by HoloInvert:2.0b.f (Prog. 2.10) is window.in, which merely specifies the half-angle size a of the window function  in degrees.

The main numerical evaluation subroutine of the reconstruction algorithm of Method B (Eq. (1.11)) then becomes:

,				  (2.22)

where  is an if-then conditional statement that allows the  contribution to the numerical integral summation to be non-zero if the angle  is smaller than the window half-angle a.  The included cone is here abruptly terminated at its edge, with no smooth transition between included points and excluded points.  Although this can introduce additional fine structure in the resulting transform, it is the method used in prior applications of this method to photoelectron diffraction data.

Method C

The additional data file read in by HoloInvert:2.0c.f (Prog. 2.11) is rous.in, which is the image intensity  output from HoloInvert.f.

The main numerical evaluation subroutine of the reconstruction algorithm of Method C (Eq. (1.12)) is identical to Method A (Eq. (1.9)), but image intensities are evaluated for points  that lie radially outward from the origin, where dr' º  and a fixed small number.  Thus image  intensities are computed that can be used together with to implement the reconstruction algorithm of Method C:

.					  (2.23)

Step sizes dr' were varied, and found to have little effect on image reconstructed via Method C, as long as dr' was smaller than the finest imagible features desired.

Method D

Similar to HoloInvert:2.0a.f, the additional data file read in by HoloInvert:2.0d.f (Prog. 2.12) is fk.in.

The main numerical evaluation subroutine of the reconstruction algorithm of Method D (Eq. (1.13)) is different than those of Methods A-C:

= θcosθφexpr'kχ(k,θ,φ)χtheoretical(k,θ,φ,x',y',z'),		  (2.24)

where the single-scattering calculated intensities are given by

χtheoretical(k,θ,φ,x',y',z')≡Refk(Θr'k)ei(k⋅r'−kr')kr'.					  (2.25)

The cosq factor is here preserved from the volume element in k-space that is present in the other methods in order to provide some weighting based on the solid angle encompassed by each direction, but in keeping with the form used by Hofmann and Schindler in their original proposal of this method [2.25], there is no k2 factor in the sum on k.

2.7  Graphical presentation of reconstructed atomic images

All of the reconstructed images in this work have been processed by HoloGraph.m, a MathematicaÓ macro developed by Budge [2.26] that plots these images as PostscriptÓ files [2.27].  These Postscript files are then translated into PICT format using GhostviewÓ [2.28], cropped with GraphicConverterÓ [2.29], and then realized in their final form using ClarisDrawÓ to add labels, axes, scales, etc. [2.30].

2.8  Concluding remarks

In this chapter we have given a brief overview of the basic theoretical methodology and the resulting computer programs used in this study of atomic x-ray and electron holography, as well as more specific instructions and examples for the programs that were developed especially for this work.

References

[2.1]	D. J. Friedman and C. S. Fadley, J. Electron Spectrosc. Relat. Phenom. 57 (1991) 223.

[2.2]	M. Sagurton, E. L. Bullock, and C. S. Fadley, Surf. Sci. 182 (1987) 287.

[2.3]	Y. Chen, H. Wu, and D. A. Shirley, private communication.

[2.4]	J. J. Rehr and R. C. Albers, Phys. Rev. B 41 (1990) 8139.

[2.5]	A. P. Kaduwela, D. J. Friedman, and C. S. Fadley, J. Electron. Spectrosc. Relat. Phenom. 57 (1991) 223.

[2.6]	R. X. Ynzunza, unpublished results.

[2.7]	M. T. Manson and S. Skillman.

[2.8]	S. M. Goldberg, C. S. Fadley, and S. Kono, J. Electron Spect. Relat. Phenom. 21 (1981) 285.

[2.9]	M. P. Seah and W. A. Dench, Surf. Interface Anal. 1 (1977) 2.

[2.10]	T. L. Loucks, Augmented Plane Wave Method (W. A. Benjamin, Inc., New York, 1967).

[2.11]	A. W. Zangwill, Physics at Surfaces (Cambridge University Press, Cambridge, 1988) 171.

[2.12]	M. Tegze and G. Faigel, Europhys. Lett. 16 (1991) 41, and Nature 380 (1996) 49.

[2.13]	(a) T. Gog, P. M. Len, G. Materlik, D. Bahr, C. Sanchez-Hanke, and C. S. Fadley, Phys. Rev. Lett. 76 (1996) 3132.  (b) P. M. Len, T. Gog, D. Novikov, R. A. Eisenhower, G. Materlik, and C. S. Fadley, submitted to Phys. Rev. B.

[2.14]	D. Novikov, private communication.

[2.15]	B.W. Batterman and H. Cole, Rev. Mod. Phys. 36 (1964) 681.

[2.16]	International Tables for X-ray Crystallography, edited by K. Lonsdale (Reidel, Dordrecht, 1968), Vol. III.

[2.17]	Wolfram Research Inc.

[2.18]	SunOS release 4.1, Sun Microsystems Inc.

[2.19]	L. V. Azaroff, Elements of X-ray Crystallography (McGraw-Hill, New York, 1968), 87.

[2.20]	L. J. Terminello, B. L. Petersen, and J. J. Barton, J. Electron Spectrosc. Relat. Phenom. 75 (1995) 229, and references therein.

[2.21]	C. M. Wei, J. H. Hong, P. R. Jeng, S. C. Shyu, and Y. C. Chou, Chem. Phys. Lett. 228 (1994) 513, and references therein.

[2.22]	G. R. Harp, D. K. Saldin, X. Chen, Z.-L. Han, and B. P. Tonner, J. Electron Spectrosc. Relat. Phenom. 57 (1991) 331.

[2.23]	R. Denecke, R. Eckstein, L. Ley, A. E. Bocquet, J. D. Riley, and R. C. G. Leckey, Surf. Sci. 331-333 (1995) 1085.

[2.24]	S. D. Ruebush, Ph. D. dissertation, University of California, Davis, CA, June 1996.

[2.25]	O. Schaff, P. Hofmann, C. J. Hirschmugl, A. Theobald, V. Fernandez, K.-M. Schindler, A. M. Bradshaw, N. Booth, R. Davis, and D. P. Woodruff, J. Electron Spec. Relat. Phenom. 76 (1995) 85, and references therein.

[2.26]	A. Budge, private communication.

[2.27]	Adobe Systems, Inc.

[2.28]	L. P. Deutsch and M. Lentczner, Aladdin Enterprises.

[2.29]	T. Lemke, Lemke Software.

[2.30]	Claris Corporation.

Figure captions

Figure 2.1.  (a) Schematic representation of the direct, singly-scattered, and Nthly-scattered wave propagation contributions to the diffraction intensity measured by the far field detector at ¥.  (b) Schematic representation of the seperable Green's function expression for the Nthly-scattered wave propagator of (a).

Figure 2.2.  Scattering factor magnitudes for k = 5.220 (E = 10.30keV) x-rays incident on atomic Fe.  Q is the angle between the incident (k) and scattered (k') wavevectors.  (a) Fe atomic scattering factor, in relative units of electrons.  (b)-(d) Thomson electron scattering factors, in absolute units of  =  º , for (b) unpolarized, (c) vertically polarized, and (d) horizontally polarized incident x-rays.

Figure 2.3.  Schematic k-space volume representations of theoretical data sets for surface W  emission, as viewed down along  (top panels), and down along  (bottom panels).  The intensities in the lower right-hand quadrant have been removed to show the intensity surface at the constant minimum wavenumber.  (a) Raw theoretical I(k) data set.  (b) Theoretical  background.  (c) Theoretical normalized  data set.

Figure 2.4.  As Fig. 2.3, but for (a) Theoretical  background (cf. Fig. 2.3(b)).  (b)  as determined by a least-squares  fit in wavenumber and polar angle of Eq. (2.19) to the raw I(k) intensities of Fig. 2.3(a).  (c)  as determined by a least-squares  fit in wavenumber and polar angle of Eq. (2.19) to the raw I(k) intensities of Fig. 2.3(a).

Figure 2.5.  As Fig. 2.4, but for (b) , and (c)  fits.